Emulating a Hotel Room Key:

- Often fails because modern access systems use encryption or rolling codes that prevent simple “read and play” duplication.

  - If the emulation is not working it is likely due to:

    - The card is encrypted (MIFARE Classic)

      - Most hotel cards are MIFARE Classic cards.

      - To successfully emulate them, you must read all sectors.

        - Example: 32/32 keys for a 1k card

  - If your flipper only readds the UID, the door will not unlock because it requires the encrypted data stored in the card’s sectors

  - You need to interrogate the door reader to find missing keys.

    - Use the detect reader feature on your Flipper, then run the MFkey32 tool via the mobile app to calculate the necessary keys.

  - Note: Some users on Reddit have noted that even with all sectors read, certain high-security systems like Salto or Saflok may still fail to emulate.

    - Saflok may fail to emulate.

- Rolling codes or counters:

  - Some advanced hotel systems use rolling codes.

    - Every time you tab your card, the reader writes a new counter value or a small piece of data back to the physical card.

      - Why it fails:

        - Since the flipper zero emulates a static snapshot of the card, the counter on your flipper will eventually mismatch what the door expects, causing the emulation to stop working.

          - Risk: In some cases, attempting to emulate these cards can actually desynchronize your original physical key, making it stop working as well.

- Technology mismatch:

  - You might be using the wrong application for the card type.

  - NFC vs. RFID: Most modern cards are NFC (13.56 MHz), but some older systems still use 125 kHz FRID

  - If the key has a black magnetic strip and no internal chip, the Flipper Zero cannot read or emulate it directly as it does not have a magnetic stripe reader/writer.

- Hardware positioning issue:

  - Alignment: Ensure you are holding the card near the center of the Flipper’s back

    - Some cards require a small gap of about 1 cm for a clean read.

  - SD card: High quality microSD card is required to store the database used for cracking keys.

    - If your SD card is missing or corrupted, the Flipper may fail to decrypt the card.

  - Firmware: Some users on Facebook have suggested that specific firmware versions can affect NFC emulation stability.

  - Ensure your device is up to date using the official Flipper firmware update tool.

- To get an encrypted card working, you have to follow a multi-step process to collect the secret keys from the door reader itself.

  - Here is the proper flow to use the MfKey32 attack:

    - Initial Read & Save

    - Go to NFC  Read and scan your hotel card.

      - It is likely that it won’t find all keys (e.g., it might say 2/32 found).

      - Save the cad anyway and give it a name.

    - Collect Nonces (Interrogate the Reader)

    - Go to NFC  Saved and select the card you just saved

    - Select Extract MF keys (or Detect Reader on some firmware versions)

    - Hold your Flipper up to the hotel door lock

      - You may need to tap it against the reader several times until the Flipper says it has collected enough nonces.

        - Usually 10 pairs

    - Calculate the keys

      - Once you have th nonces, you nee to calculate the actual keys.

        - You can do this int wo ways:

          - On the Flipper:

            - Go to Apps  NFC  MFkey and run it.

              - This is slower but doesn’t require a phone

    - On your phone:

      - Open the Flipper Mobile App, sync your device, and go to HUB  NFC tools  MFKeys32

        - This is usually much faster because your phone has more processing power

    - Re-read the card:

      - After the MFkey app finishes, it will automatically add the new keys to your Flipper’s User Dictionary

    - Go back to NFC  Read and scan your physical hotel card again.

      - This time, it should find all keys (32/32 for standard 1

    - Emulate:

      - Save this new, complete scan

        - Now you can select emulate and the door should unlock

        - For more detailed technical steps, you can check the recovering MIFARE Classic Keys